Supplementary Materials for

Single and Double-sided Coated Gas Diffusion Layers used in Polymer Electrolyte Fuel Cells: A Numerical Study

I. C. Okereke a, b, M.S. Ismail c, D.B. Ingham a, K.J. Hughes a*, L. Ma a, M. Pourkashanian a, d

a Energy Institute, The University of Sheffield, Sheffield, UK.

b Department of Mechanical Engineering, Faculty of Engineering, Akwa Ibom State University, Nigeria.

c School of Engineering, University of Hull, Hull HU6 7RX, UK

d Translational Energy Research Centre, The University of Sheffield, Sheffield, UK.

Corresponding author: K.J.Hughes@sheffield.ac.uk (K.J. Hughes)

Source terms used in the transport equations:

- Mass transport:

Smass= 0. Where Smass is the mass source term

- Momentum transport:

Smom= μuK

where Smom is the momentum source, μ is the dynamic viscosity, u is the velocity vector, and Kis the permeability.

- Species transport:

Sk is the source term that represents either consumption/production of species k (H2, O2 or H2O) and is given as follows (Ismail et al., 2012, Okereke et al.,2023):

SH2= -iaaa2FMH2

SO2= -icac4FMO2

SH2O= icac2FMH2O

where ia and ic are the anodic and cathodic local current density respectively, aa and ac are the anodic and cathodic specific surface areas respectively, F is the Faraday’s constant (96485 C/mol), and MH2, MO2 and MH2O are the molecular weights for hydrogen, oxygen and water respectively.

- Energy transport:

Se is the heat source term and takes one of the following forms in each fuel cell component (Li et al., 2017):

Se= is2σs for anode and cathode GDLsRaηa-T∆Sa2F+is2σs+im2σm for anode catalyst layerRc-ηc-T∆Sc2F+is2σs+im2σm for cathode catalyst layer

where Ra and Rc are the anode and cathode exchange current densities, is and im are the solid phase and membrane phase current densities, σs and σm are the electrical and ionic conductivities of the solid and membrane phases respectively, and ηa and ηc are the anodic and cathodic overpotential, ∆Sa and ∆Sc are the reaction entropies at anode and cathode catalyst layers respectively.

5. Charge transport:

Sϕ,s and Sϕ,m are the solid-phase potential and membrane-phase potential respectively and are given as follows (Alhazmi et al., 2013):

Sϕ,s=ja at the anode CL-jc at the cathode CL

Sϕ,m=-ja at the anode CLjc at the cathode CL

where ja and jc are the volumetric exchange current density (A/m³) at the anode and cathode catalyst layers respectively and are obtained using Butler-Volmer equations (Zawodzinski et al., 1993):

ja= iarefaacH2cH2ref0.5expαa,aFRTηact,a-expαa,cFRTηact,a

jc= icrefaccO2cO2refexpαc,aFRTηact,c-expαc,cFRTηact,c

where iarefand icref are the reference anodic and cathodic exchange current density respectively, αa,a and αa,c are respectively the anode and cathode transfer coefficients for the electrochemical reactions in the anode catalyst layer, αc,a and αc,c are respectively the anode and cathode transfer coefficients in the cathode catalyst layer, cH2ref and cO2refare the reference hydrogen and oxygen concentrations respectively, F is the Faraday’s constant and R is the universal gas constant. ηact,a and ηact,c are the anodic and cathodic overpotential and are given as follows:

ηact,a= ϕs-ϕm

ηact,c= ϕs-ϕm-E0

where E0 is the reference potential of the electrodes and is equal to zero for the anode, while for the cathode it is equal to the equilibrium cell potential (Er) (Gostick et al., 2006; Pharaoh et al., 2006):

Er=1.482-0.000845T+0.0000431Tln PH2PO20.5

The membrane ionic conductivity, σm, is estimated using an empirical correlation developed by Springer et al. (1991):

ϕm=(0.005139λ -0.00326) exp12681303-1T

where λ is the membrane water content which is empirically correlated by Zawodzinski et al. (1993):

λ= 1.409+11.26ac-18.77ac2+16.21ac3, 0<ac≤110.11+2.944ac-1, 1<ac≤3                                   16.8,  ac>3

where ac  is the water activity and is given as (Ismail et al., 2012):

ac=PvPs

where Pv is the partial pressure of water vapour and Ps is the pressure of saturated water vapor which is given by (Ismail et al., 2012):

log(Ps)=-2.1794+0.02953(T-273.15)-9.1837×10-5(T-273.15)2+1.4454×10-7(T-273.15)3

6. Liquid water transport:

Sl  is the source for the liquid phase, and is specified as follows (Li et al., 2017):

Sl= Sphase-SλMH2O+ic2FMH2O at the cathode catalyst layer.

Sl= Sphase at the anode catalyst layer, anode gas diffusion layer, and the cathode gas diffusion layer given as follows:

Sphase= Kcondε1-sxvRTMH2OPv-Ps                     Pv≥Ps KevapεsρlPv-Ps                         Pv<Ps

where Kcond  is the rate of condensation, Kevap is the rate of evaporation, ρl is the density of liquid water, and xv is the mole fraction of water vapour.

Sλ= γaρmMm(λeq-λ) at the anode catalyst layer

Sλ= γcρmMm(λeq-λ)+ic2F at the cathode catalyst layer

where ρm is the density of the dry membrane and Mm is the membrane equivalent weight.