Supplementary Materials: Conceptualization and construction of a low-cost and self-made device for monitoring of Particulate Matter: a step-by-step guide

Giacomo Fanti1,*, Francesca Borghi1,3, Emanuele Cauda4, Cody Wolfe4, Justin Patts4, Carlo Dossi2, Andrea Cattaneo1, Andrea Spinazzè1, Domenico Maria Cavallo1

- Department of Science and High Technology, University of Insubria, Como, Italy;

francesca.borghi@uninsubria.it (F.B.); andrea.cattaneo@uninsubria.it (A.C.);

andrea.spinazze@uninsubria.it (A.S.); domenico.cavallo@uninsubria.it (D.M.C.)

- Department of Theoretical and Applied Sciences, University of Insubria, Varese, Italy;

Carlo.Dossi@uninsubria.it (C.D.)

- Department of Medical and Surgical Sciences, University of Bologna, Bologna, Italy;

francesca.borghi12@unibo.it (F.B.)

- Center for Direct Reading and Sensor Technologies, National Institute for Occupational Safety and Health, Centers for Disease Control and Prevention, Pittsburgh, Pennsylvania, PA, USA;

cuu5@cdc.gov (E.C.); yvv5@cdc.gov (C.W.); jdq7@cdc.gov (J.P.)

*	Correspondence: giacomo.fanti@uninsubria.it (G.F.) Tel.: +39-0312386645

# Script S1: Arduino IDE sketch (text file)

#include <SoftwareSerial.h>

SoftwareSerial pmsSerial(2, 3);

#include <SPI.h>

#include <SD.h>

#include <SimpleDHT.h>

#define _DHTPIN 4

SimpleDHT22 dht22(_DHTPIN);

#include <Wire.h>

#include <RTClib.h>

RTC_DS3231 rtc;

int intestaColonne = 0;

void setup() {

// our debugging output

Serial.begin(115200);

// sensor baud rate is 9600

pmsSerial.begin(9600);

if (!SD.begin(10)){

Serial.println("errore SD");

while(1);

}

File dataFile = SD.open ("P.ALP.txt", FILE_WRITE);

if (dataFile) {

dataFile.print ("Date"); dataFile.print (";"); dataFile.print ("Time"); dataFile.print (";"); dataFile.print("PM1"); dataFile.print(";"); dataFile.print("PM2.5"); dataFile.print(";"); dataFile.print("PM10"); dataFile.print(";");

dataFile.print("Npart_0.3um"); dataFile.print(";"); dataFile.print("Npart_0.5um"); dataFile.print(";"); dataFile.print("Npart_1um"); dataFile.print(";");

dataFile.print("Npart_2.5um"); dataFile.print(";"); dataFile.print("Npart_5um"); dataFile.print(";"); dataFile.print("Npart_10um"); dataFile.print(";");

dataFile.print("T"); dataFile.print(";"); dataFile.print("RH"); dataFile.println("\t");

dataFile.close();

}

if (!rtc.begin()){

Serial.println("verify connections");

while(true);

}

if (rtc.begin()) {

rtc.adjust(DateTime(F(__DATE__), F(__TIME__)));

//rtc.adjust(DateTime(2021, 11, 22, 12, 00));

}

}

struct pms5003data {

uint16_t framelen;

uint16_t pm10_standard, pm25_standard, pm100_standard;

uint16_t pm10_env, pm25_env, pm100_env;

uint16_t particles_03um, particles_05um, particles_10um, particles_25um, particles_50um, particles_100um;

uint16_t unused;

uint16_t checksum;

};

struct pms5003data data;

void loop() {

float temp = 0;

float hum = 0;

dht22.read2(&temp, &hum, NULL);

if (!intestaColonne) {

Serial.print("\t"); Serial.print("Data e Ora"); Serial.print("\t"); Serial.print("\t"); Serial.print("PM1"); Serial.print("\t"); Serial.print("PM2.5"); Serial.print("\t"); Serial.print("PM10"); Serial.print("\t");

Serial.print("Npart_0.3um"); Serial.print("\t"); Serial.print("Npart_0.5um"); Serial.print("\t"); Serial.print("Npart_1um"); Serial.print("\t");

Serial.print("Npart_2.5um"); Serial.print("\t"); Serial.print("Npart_5um"); Serial.print("\t"); Serial.print("Npart_10um"); Serial.print("\t");

Serial.print("Temperature"); Serial.print("\t"); Serial.print("Humidity"); Serial.println("\t");

intestaColonne = 1;

}

DateTime now = rtc.now();

if (readPMSdata(&pmsSerial)) {

// reading data was successful!

Serial.print("\t"); Serial.print (now.year(), DEC); Serial.print ("/"); Serial.print (pad(now.month())); Serial.print ("/"); Serial.print (pad(now.day())); Serial.print (" "); Serial.print (pad(now.hour())); Serial.print (":"); Serial.print (pad(now.minute())); Serial.print (":"); Serial.print (pad(now.second())); Serial.print("\t"); Serial.print(data.pm10_standard); Serial.print("\t"); Serial.print(data.pm25_standard); Serial.print("\t"); Serial.print(data.pm100_standard); Serial.print("\t");

Serial.print(data.particles_03um); Serial.print("\t"); Serial.print("\t"); Serial.print(data.particles_05um); Serial.print("\t"); Serial.print("\t"); Serial.print(data.particles_10um); Serial.print("\t"); Serial.print("\t");

Serial.print(data.particles_25um); Serial.print("\t"); Serial.print("\t"); Serial.print(data.particles_50um); Serial.print("\t"); Serial.print("\t"); Serial.print(data.particles_100um); Serial.print("\t"); Serial.print("\t");

Serial.print(temp); Serial.print("\t"); Serial.print("\t"); Serial.print(hum); Serial.println("\t");

}

File dataFile = SD.open ("P.ALP.txt", FILE_WRITE);

if (dataFile) {

dataFile.print (pad(now.month())); dataFile.print ("/"); dataFile.print (pad(now.day())); dataFile.print ("/"); dataFile.print (now.year(), DEC); dataFile.print (";"); dataFile.print (pad(now.hour())); dataFile.print (":"); dataFile.print (pad(now.minute())); dataFile.print (":"); dataFile.print (pad(now.second())); dataFile.print (";"); dataFile.print(data.pm10_standard); dataFile.print(";"); dataFile.print(data.pm25_standard); dataFile.print(";"); dataFile.print(data.pm100_standard); dataFile.print(";");

dataFile.print(data.particles_03um); dataFile.print(";"); dataFile.print(data.particles_05um); dataFile.print(";"); dataFile.print(data.particles_10um); dataFile.print(";");

dataFile.print(data.particles_25um); dataFile.print(";"); dataFile.print(data.particles_50um); dataFile.print(";"); dataFile.print(data.particles_100um); dataFile.print(";");

dataFile.print(temp); dataFile.print(";"); dataFile.print(hum); dataFile.println("\t");

dataFile.close();

}

else {

Serial.println("error opening P.ALP.txt");

}

}

char *res = malloc (5);

String pad(int n) {

sprintf(res, "%02d", n);

return String(res);

}

boolean readPMSdata(Stream *s) {

if (! s->available()) {

return false;

}

// Read a byte at a time until we get to the special '0x42' start-byte

if (s->peek() != 0x42) {

s->read();

return false;

}

if (s->available() < 32) {

return false;

}

uint8_t buffer[32];

uint16_t sum = 0;

s->readBytes(buffer, 32);

for (uint8_t i=0; i<30; i++) {

sum += buffer[i];

}

/* debugging

for (uint8_t i=2; i<32; i++) {

Serial.print("0x"); Serial.print(buffer[i], HEX); Serial.print(", ");

}

Serial.println();

*/

// The data comes in endian'd, this solves it so it works on all platforms

uint16_t buffer_u16[15];

for (uint8_t i=0; i<15; i++) {

buffer_u16[i] = buffer[2 + i*2 + 1];

buffer_u16[i] += (buffer[2 + i*2] << 8);

}

// put it into a nice struct :)

memcpy((void *)&data, (void *)buffer_u16, 30);

if (sum != data.checksum) {

Serial.println("Checksum failure");

return false;

}

// success!

return true;

}

# Script S2: R-code (text file)

###   R script to handle .txt files from the P.ALP and convert the data into a usable .csv file   ###

###   Last version 6/23/2022   ###

###   File From Folders

###   setwd(dir = "C:/example")

### REMOVE THE 3 “###” HEADING THE PREVIOUS 2 LINES IF YOU WANT TO USE THE FOLDER AS DIRECTORY”

###   File From SD card

###    setwd(dir = "E:")

### REMOVE THE 3 “###” HEADING THE PREVIOUS 2 LINES IF YOU WANT TO USE THE SD-CARD AS DIRECTORY”

CW <- read.table("P.ALP.TXT", header = TRUE, sep = ";", dec = ".")

naming_convention <- "P.ALP Session_"

###   In the following two lines you need to insert the correct date (MM/DD/YYYY) and time (HH:MM:SS)

start_date <- "01/25/2023"

start_time <- "15:37:45"

library("anytime")

library("lubridate")

#conversion from original date to current date and time

##CW$Date <- c(start_date)

start_dateTime <- as.data.frame(paste(start_date, start_time))

colnames(start_dateTime)[1]  <- "start_dateTime"

start_dateTime <- start_dateTime$start_dateTime[ 1]

badDate <- CW$Date[ 1]

badTime <- CW$Time[ 1]

times <- as.data.frame(paste(CW$Date, CW$Time))

colnames(times)[1]  <- "updatedTime"

badDateTime <- times$updatedTime[ 1]

#calculate difference between times and add to the times variable and bind to CW

good <- anytime(start_dateTime)

bad <- anytime(badDateTime)

difference <- paste(difftime(bad, good, units="secs"))

times2 <- anytime(times$updatedTime)

updatedTime <- times2 - as.numeric(difference)

times3 <- cbind(updatedTime, times)

#

CW <- cbind(times3$updatedTime, CW)

colnames(CW)[1]  <- "updatedTime"

colnames(CW)[13]  <- "Temp"

start_Time <- CW$updatedTime[1]

end_Time <- tail(CW$updatedTime, n=1)

unique_seconds <- as.data.frame(unique(CW$Time))

colnames(unique_seconds)[1]  <- "unique_seconds"

unique_minutes  <- as.data.frame(unique(substr(CW$Time,1,5)))

colnames(unique_minutes)[1]  <- "unique_minutes"

unique_hours <- as.data.frame(unique(substr(CW$Time,1,2)))

colnames(unique_hours)[1]  <- "unique_hours"

total_sampling_minutes <- nrow(unique_seconds)/60

# 1 second resolution

PM1 <- as.data.frame(tapply(CW$PM1, CW$updatedTime, mean))

colnames(PM1)[1]  <- "PM1"

PM2.5 <- as.data.frame(tapply(CW$PM2.5, CW$updatedTime, mean))

colnames(PM2.5)[1]  <- "PM2.5"

PM10 <- as.data.frame(tapply(CW$PM10, CW$updatedTime, mean))

colnames(PM10)[1]  <- "PM10"

Npart_0.3um <- as.data.frame(tapply(CW$Npart_0.3um, CW$updatedTime, mean))

colnames(Npart_0.3um)[1]  <- "Npart_0.3um"

Npart_0.5um <- as.data.frame(tapply(CW$Npart_0.5um, CW$updatedTime, mean))

colnames(Npart_0.5um)[1]  <- "Npart_0.5um"

Npart_1um <- as.data.frame(tapply(CW$Npart_1um, CW$updatedTime, mean))

colnames(Npart_1um)[1]  <- "Npart_1um"

Npart_2.5um <- as.data.frame(tapply(CW$Npart_2.5um, CW$updatedTime, mean))

colnames(Npart_2.5um)[1]  <- "Npart_2.5um"

Npart_5um <- as.data.frame(tapply(CW$Npart_5um, CW$updatedTime, mean))

colnames(Npart_5um)[1]  <- "Npart_5um"

Npart_10um <- as.data.frame(tapply(CW$Npart_10um, CW$updatedTime, mean))

colnames(Npart_10um)[1]  <- "Npart_10um"

CW[, 13:14][CW[, 13:14] == 0] <- NA

Temp <- as.data.frame(tapply(CW$Temp, CW$updatedTime, mean, na.rm = TRUE))

colnames(Temp)[1]  <- "Temp"

RH <- as.data.frame(tapply(CW$RH, CW$updatedTime, mean, na.rm = TRUE))

colnames(RH)[1]  <- "RH"

second_resolution <- cbind(PM1,PM2.5,PM10,Npart_0.3um,Npart_0.5um,Npart_1um,Npart_2.5um,Npart_5um,Npart_10um,Temp,RH)

second_resolution_round <- round(second_resolution, digits = 1)

write.csv(second_resolution_round, paste0(naming_convention,"AT_1sec.csv"))

# 1 min resolution

times_min <- as.data.frame(substr(CW$updatedTime,1,16))

colnames(times_min)[1]  <- "times_min"

CW <- cbind(times_min, CW)

PM1 <- as.data.frame(tapply(CW$PM1, CW$times_min, mean))

colnames(PM1)[1]  <- "PM1"

PM2.5 <- as.data.frame(tapply(CW$PM2.5, CW$times_min, mean))

colnames(PM2.5)[1]  <- "PM2.5"

PM10 <- as.data.frame(tapply(CW$PM10, CW$times_min, mean))

colnames(PM10)[1]  <- "PM10"

Npart_0.3um <- as.data.frame(tapply(CW$Npart_0.3um, CW$times_min, mean))

colnames(Npart_0.3um)[1]  <- "Npart_0.3um"

Npart_0.5um <- as.data.frame(tapply(CW$Npart_0.5um, CW$times_min, mean))

colnames(Npart_0.5um)[1]  <- "Npart_0.5um"

Npart_1um <- as.data.frame(tapply(CW$Npart_1um, CW$times_min, mean))

colnames(Npart_1um)[1]  <- "Npart_1um"

Npart_2.5um <- as.data.frame(tapply(CW$Npart_2.5um, CW$times_min, mean))

colnames(Npart_2.5um)[1]  <- "Npart_2.5um"

Npart_5um <- as.data.frame(tapply(CW$Npart_5um, CW$times_min, mean))

colnames(Npart_5um)[1]  <- "Npart_5um"

Npart_10um <- as.data.frame(tapply(CW$Npart_10um, CW$times_min, mean))

colnames(Npart_10um)[1]  <- "Npart_10um"

CW[, 14:15][CW[, 14:15] == 0] <- NA

Temp <- as.data.frame(tapply(CW$Temp, CW$times_min, mean, na.rm = TRUE))

colnames(Temp)[1]  <- "Temp"

RH <- as.data.frame(tapply(CW$RH, CW$times_min, mean, na.rm = TRUE))

colnames(RH)[1]  <- "RH"

min_resolution <- cbind(PM1,PM2.5,PM10,Npart_0.3um,Npart_0.5um,Npart_1um,Npart_2.5um,Npart_5um,Npart_10um,Temp,RH)

min_resolution_round <- round(min_resolution, digits = 1)

write.csv(min_resolution_round, paste0(naming_convention,"AT_1min.csv"))

total_sampling_minutes

###   END   ###

# Additional Figures

Figure S1: Arduino Uno-R3 microcontroller board

Figure S2: Elegoo Uno-R3 microcontroller board

Figure S3: Plantower PMS5003 Sensor for Particulate Matter monitoring

Figure S4: DHT22 sensor

Figure S5: DS3231 Real Time Clock module

Figure S6: Micro-SD card adapter module

Figure S7: Micro-SD card.

Figure S8: Prototype breadboard

Figure S9: Solderable board

Figure S10: USB type C (female) to USB type B (male) converter

Figure S11: USB type C (male) to USB type A (male) connection cable

Figure S12: 10400mAh power bank battery

Figure S13: Jumper wires Elegoo kit

Figure S14: Airtight container

Figure S15: Airtight box with the P.ALP’s environmental sensors